# Heliostat Field Optimization Framework – Architecture Overview

# 1. PROJECT DEFINITION LAYER

This layer defines the optimization project and its configuration:

- project_config.json: Contains fields such as:

  - project_name

  - location.latitude, location.longitude

  - data.dni_file, data.directions_with_weights_file

  - executables: Paths to Tonatiuh++, Energy, and Sampling tools

  - optimization_config: Population size, bounds, parameters, etc.

- ProjectManager: Python class that parses the JSON config and provides properties to access file paths and parameters.

# 2. OPTIMIZATION FRAMEWORK LAYER

Encapsulates the control logic of the optimizer in a flexible and extensible way. The logic follows a standard metaheuristic interface: suggest → evaluate → update → is_done

## Key Classes and Files:

| Component | Purpose |
| --- | --- |
| MetaheuristicOptimizer | Abstract base class defining suggest, update, is_done |
| DifferentialEvolutionOptimizer | Implementation of DE algorithm |
| optimizer_runner.py | Driver script that loops through the optimization cycle |

# 3. LAYOUT GENERATION SYSTEM

Defines a modular system for generating heliostat field layouts using parameterized methods.

Key Classes and Files:

| Component | Purpose |
| --- | --- |
| ParametricLayoutGenerator | Abstract base class: exposes generate_layout(output_file, parameters) |
| BiomimeticSpiralGenerator | Concrete class implementing the biomimetic spiral layout |
| layout_generator_factory.py | Selects appropriate generator from config (e.g., biomimetic, radial) |

The generator receives a parameter set and outputs a .csv file compliant with the Tonatiuh++ layout script.

# 4. FIREWORKS WORKFLOW LAYER

Each layout evaluation is a black-box process managed via FireWorks. Each step is encapsulated in a FireTask.

## Key FireTasks:

| Task | Function |
| --- | --- |
| generate_layout_from_parameters | Uses selected layout generator to create the .csv layout |
| run_tonatiuh_simulation | Launches Tonatiuh++ with the layout file |
| compute_annual_energy | Runs the compiled AnnualEnergy.exe tool to compute energy and efficiency |
| compute_fitness | Records fitness result and updates parameter_sets.csv |
| create_population_folder | Creates a new folder named population_XXX for each generation |

These tasks are grouped into FireWorks workflows that can run in parallel.

# 5. DATA MANAGEMENT STRUCTURE

Each project has a structured directory format:

sql

CopyEdit

projects/

tarancon_spain/

data/

dni_tarancon_spain.csv

directions_with_weights_tarancon_spain.csv

parameter_sets.csv          ← Appended with each generation

layouts/

population_000/

layout_H000.csv

layout_H001.csv

...

population_001/

...

results/

tarancon_spain_annual_energy.csv

Each generation produces one folder in layouts/. All parameter sets and fitness values are collected in a single parameter_sets.csv.

# 6. OPTIMIZATION WORKFLOW EXECUTION

Each iteration (generation) consists of:

- Suggest
The optimizer proposes a set of parameter sets for the new population.

- Evaluate
Each parameter set is evaluated by:

  - Generating a layout

  - Running Tonatiuh++

  - Computing energy and fitness

- Update
The optimizer receives the fitness values and updates its internal state.

- Repeat
Until convergence conditions are met (e.g., max_generations, no improvement).

# 7. MODULES TO BE DEVELOPED

| Module/File | Description |
| --- | --- |
| metaheuristics/metaheuristic_optimizer.py | Base class for any metaheuristic algorithm |
| metaheuristics/de_optimizer.py | Implementation of Differential Evolution |
| metaheuristics/optimizer_runner.py | Controls the optimization loop |
| layout_generators/layout_generator_base.py | Interface for layout generators |
| layout_generators/biomimetic_spiral.py | Biomimetic spiral layout generator class |
| layout_generators/factory.py | Returns the correct generator based on project config |
| firetasks/generate_layout_from_parameters.py | Generates layout from parameter set and stores layout CSV |
| firetasks/compute_fitness.py | Computes fitness and stores result in parameter_sets.csv |
| firetasks/create_population_folder.py | Creates the appropriate generation folder under layouts/ |
| workflows/biomimetic_optimization_workflow.py | Assembles the full FireWorks optimization workflow |